EXPERIMENT 02

clc;

clear;

close all;

%% MICROWAVE TRANSMISSION LINES AND TWO-PORT NETWORKS

% S-Parameter Analysis

c = 3e8;                    % Speed of light (m/s)

er = 2.1;                   % Relative permittivity

Zref = 50;                  % Reference impedance (ohms)

% Coaxial transmission line dimensions

a_inner = 0.5e-3;           % Inner conductor radius (m)

b_outer = 1.65e-3;          % Outer conductor inner radius (m)

L = 0.2;                    % Transmission line length (m)

%% OBJECTIVE 1: MODEL THE COAXIAL TRANSMISSION LINE

x = linspace(-b_outer,b_outer,100);

y = linspace(-b_outer,b_outer,100);

[X,Y] = meshgrid(x,y);

R = sqrt(X.^2 + Y.^2);

% Electric field for an assumed 1 V potential difference

E = 1 ./ (R .* log(b_outer/a_inner));

% Field exists only in the dielectric region

E(R < a_inner | R > b_outer) = NaN;

%% OBJECTIVE 2: CHARACTERISTIC IMPEDANCE AND PROPAGATION CONSTANT

% Approximate characteristic impedance of a coaxial line

Z0 = (60/sqrt(er))*log(b_outer/a_inner);

% Frequency range: 1 GHz to 10 GHz

f = linspace(1e9,10e9,600);

w = 2*pi*f;

% Phase and attenuation constants

beta = w*sqrt(er)/c;              % rad/m

% Illustrative conductor and dielectric losses

alpha_c = 0.035*sqrt(f/1e9);       % Np/m

tan_delta = 0.001;

alpha_d = beta*tan_delta/2;        % Np/m

alpha = alpha_c + alpha_d;

gamma = alpha + 1i*beta;

fprintf('TRANSMISSION LINE PARAMETERS\n');

fprintf('----------------------------\n');

fprintf('Characteristic impedance = %.3f ohms\n',Z0);

fprintf('Line length = %.3f m\n',L);

fprintf('Frequency range = 1 to 10 GHz\n');

fprintf('Reference impedance = %.1f ohms\n',Zref);

%% OBJECTIVE 3: CALCULATE S-PARAMETERS

S11 = zeros(size(f));

S21 = zeros(size(f));

S12 = zeros(size(f));

S22 = zeros(size(f));

for k = 1:length(f)

% ABCD parameters of the transmission line

A = cosh(gamma(k)*L);

B = Z0*sinh(gamma(k)*L);

C = sinh(gamma(k)*L)/Z0;

D = cosh(gamma(k)*L);

den = A + B/Zref + C*Zref + D;

% Scattering parameters

S11(k) = (A + B/Zref - C*Zref - D)/den;

S21(k) = 2/den;

S12(k) = 2*(A*D - B*C)/den;

S22(k) = (-A + B/Zref - C*Zref + D)/den;

end

%% OBJECTIVE 4: RETURN LOSS, INSERTION LOSS AND EFFICIENCY

S11_dB = 20*log10(max(abs(S11),1e-12));

S21_dB = 20*log10(max(abs(S21),1e-12));

S12_dB = 20*log10(max(abs(S12),1e-12));

S22_dB = 20*log10(max(abs(S22),1e-12));

ReturnLoss = -S11_dB;

InsertionLoss = -S21_dB;

Efficiency = abs(S21).^2*100;

fprintf('\nS-PARAMETER RESULTS AT 5 GHz\n');

fprintf('----------------------------\n');

[~,k5] = min(abs(f-5e9));

fprintf('|S11| = %.4f\n',abs(S11(k5)));

fprintf('|S21| = %.4f\n',abs(S21(k5)));

fprintf('|S12| = %.4f\n',abs(S12(k5)));

fprintf('|S22| = %.4f\n',abs(S22(k5)));

fprintf('Return loss = %.3f dB\n',ReturnLoss(k5));

fprintf('Insertion loss = %.3f dB\n',InsertionLoss(k5));

fprintf('Transmission efficiency = %.2f %%\n',Efficiency(k5));

%% CREATE ONE FIGURE WITH SIX GRAPHS

figure('Name','Microwave Transmission Line S-Parameter Analysis',...

'NumberTitle','off',...

'Position',[50 50 1400 900]);

%% GRAPH 1: COAXIAL TRANSMISSION LINE MODEL

subplot(3,2,1);

surf(X*1000,Y*1000,E,'EdgeColor','none');

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('Electric Field (V/m)');

title('Graph 1: Coaxial Line Electric Field');

colorbar;

view(45,35);

grid on;

axis tight;

%% GRAPH 2: PROPAGATION CONSTANTS

subplot(3,2,2);

yyaxis left;

plot(f/1e9,beta,'b','LineWidth',2);

ylabel('Phase Constant \beta (rad/m)');

yyaxis right;

plot(f/1e9,alpha,'r--','LineWidth',2);

ylabel('Attenuation Constant \alpha (Np/m)');

xlabel('Frequency (GHz)');

title('Graph 2: Propagation Constants');

grid on;

%% GRAPH 3: MAGNITUDE OF FOUR S-PARAMETERS

subplot(3,2,3);

plot(f/1e9,S11_dB,'b','LineWidth',1.8);

hold on;

plot(f/1e9,S21_dB,'r','LineWidth',1.8);

plot(f/1e9,S12_dB,'k--','LineWidth',1.8);

plot(f/1e9,S22_dB,'m-.','LineWidth',1.8);

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('Graph 3: S-Parameter Magnitudes');

legend('S11','S21','S12','S22','Location','best');

grid on;

%% GRAPH 4: PHASE OF S-PARAMETERS

subplot(3,2,4);

plot(f/1e9,unwrap(angle(S11))*180/pi,'b','LineWidth',1.5);

hold on;

plot(f/1e9,unwrap(angle(S21))*180/pi,'r','LineWidth',1.5);

plot(f/1e9,unwrap(angle(S12))*180/pi,'k--','LineWidth',1.5);

plot(f/1e9,unwrap(angle(S22))*180/pi,'m-.','LineWidth',1.5);

xlabel('Frequency (GHz)');

ylabel('Phase (degrees)');

title('Graph 4: S-Parameter Phase');

legend('S11','S21','S12','S22','Location','best');

grid on;

%% GRAPH 5: RETURN LOSS AND INSERTION LOSS

subplot(3,2,5);

plot(f/1e9,ReturnLoss,'b','LineWidth',2);

hold on;

plot(f/1e9,InsertionLoss,'r--','LineWidth',2);

xlabel('Frequency (GHz)');

ylabel('Loss (dB)');

title('Graph 5: Return and Insertion Loss');

legend('Return Loss','Insertion Loss','Location','best');

grid on;

%% GRAPH 6: TRANSMISSION EFFICIENCY

subplot(3,2,6);

plot(f/1e9,Efficiency,'g','LineWidth',2);

xlabel('Frequency (GHz)');

ylabel('Transmission Efficiency (%)');

title('Graph 6: Transmission Efficiency');

grid on;

ylim([0 105]);

sgtitle('Numerical Simulation of Microwave Transmission Lines and Two-Port Networks');

fprintf('\nAll six graphs generated successfully.\n');